ELG 5228

TurtleBot3 Bridge Guide

Prof. Wail Gueaieb

Version: June 18, 2026

This guide covers initial setup, writing algorithms, simulation, and real robot operation.

# 1.  Initial Setup

This section walks you through the complete setup from downloading files to being ready to write your first algorithm.

## Step 1. Set Up Docker

If you have not already set up Docker for this course, follow the instructions provided on Brightspace to install Docker Desktop and pull the course Docker image. See the Docker image documentation at https://github.com/wail-uottawa/docker-ros2-elg5228 for full details. Once complete, you should be able to start the container and access the VNC interface in your browser at http://127.0.0.1:6080.

## Step 2. Verify Docker Is Working

Start your Docker container and open a terminal inside it (through the VNC interface). Run:

echo $ROS_DISTRO

This should print "humble". If it does, Docker is set up correctly.

## Step 3. Download the Bridge Files

Download the following three files from Brightspace (or the course GitHub repository) and place them in your course_dir folder. The same local folder you already use for the course that is mapped into the Docker container.

| File | Purpose |
| --- | --- |
| roslib_wrapper.py | Connection module - do not edit |
| student_template.py | Your starting point - edit this |
| bridge_test_terminal.py | Test script - verifies everything works |

If you are unsure which folder this is, open your docker-run.sh (or docker-run.bat) and look for the local path that maps to /home/ubuntu/ros2_ws/src/course_dir. Place the three files in that folder. All three files must be in the same folder together.

## Step 4. Verify Files Are Visible in Docker

Start your Docker container, open a terminal, and check the files are there:

ls ~/ros2_ws/src/course_dir/

You should see roslib_wrapper.py, student_template.py, and bridge_test_terminal.py listed. If not, check that you placed them in the correct local folder.

## Step 5. Quick Test

Run a quick check that the wrapper module loads correctly:

cd ~/ros2_ws/src/course_dir

python3 -c "import roslib_wrapper; print('OK')"

If it prints "OK", you are ready to write your algorithm.

Setup is complete. Proceed to Section 2 to learn how to write your algorithm, Section 3 to test in simulation, or Section 4 to run on the real robot.

# 2.  Writing Your Algorithm

All your work goes in the student_template.py file. Open it and find the control_loop() method, this is the only method you need to edit. The rest of the template handles connecting to the bridge, subscribing to sensors, publishing velocities, and stopping the robot on exit.

## 2.1 Where to write your code

Inside student_template.py, look for the control_loop() method. It is called automatically at 10 Hz (10 times per second). Each time it runs, you can read the latest sensor data, decide what the robot should do, and publish a velocity command.

## 2.2 Your Topics

Regardless of simulation or real robot mode, your code uses these three topics:

| Direction | Topic | Message Type |
| --- | --- | --- |
| You Robot | /student/cmd_vel | geometry_msgs/Twist |
| Robot You | /student/scan | sensor_msgs/LaserScan |
| Robot You | /student/odom | nav_msgs/Odometry |

Do NOT publish directly to /cmd_vel. Always use /student/cmd_vel

## 2.3 What data is available to you

LiDAR scan data

Access the LiDAR through self.latest_scan.ranges. This is a list of distances in meters, one per angle. Use it to detect obstacles and walls around the robot.

- self.latest_scan.ranges[0]   =  distance directly ahead

- self.latest_scan.ranges[90]  =  distance to the left

- self.latest_scan.ranges[180] =  distance behind

- self.latest_scan.ranges[270] =  distance to the right

For example, to check if there is an obstacle within 0.3 meters in front of the robot, you would check if self.latest_scan.ranges[0] < 0.3.

Odometry (position and velocity)

Access the robot’s position and velocity through self.latest_odom. This tells you where the robot is and how fast it is moving.

- Position X: self.latest_odom.pose.pose.position.x  (meters)

- Position Y: self.latest_odom.pose.pose.position.y  (meters)

- Linear velocity: self.latest_odom.twist.twist.linear.x  (m/s)

- Angular velocity: self.latest_odom.twist.twist.angular.z  (rad/s)

For example, to track how far the robot has traveled, you could store the starting position and compute the distance using the x and y values.

Publishing velocity commands

Create a Twist message, set the forward speed and turning speed, and publish it. The robot responds immediately.

- Forward speed: cmd.linear.x  - positive = forward, max ~0.22 m/s

- Turning speed: cmd.angular.z  -  positive = left, negative = right, max ~2.84 rad/s

For example, to drive forward slowly: set cmd.linear.x = 0.1 and cmd.angular.z = 0.0. To turn left in place: set cmd.linear.x = 0.0 and cmd.angular.z = 0.5.

2.4 Important rules

- Always check if self.latest_scan is None before using it, the first few timer ticks run before any sensor data arrives.

- Publish at 10 Hz or faster. The bridge watchdog stops the robot if no command arrives for 0.5 seconds.

- The template automatically stops the robot when you press Ctrl-C, you do not need to handle this yourself.

- Do not modify the imports, the __init__ method, the callbacks, or the main function.

# 3.  Simulation (Gazebo)

Use this mode to develop and test your algorithms before running on the real robot. Everything runs inside the Docker container — no physical robot, bridge, or hotspot needed.

## Step 1.  Start the Docker Container

Start your Docker container as usual for the course. Open the VNC interface in your browser at http://127.0.0.1:6080. All following commands are run inside Docker terminals.

## Step 2.  Launch Gazebo with TurtleBot3

Open a terminal inside the Docker container and run:

export TURTLEBOT3_MODEL=waffle

ros2 launch turtlebot3_gazebo turtlebot3_world.launch.py

Wait for Gazebo to fully load with the robot visible. Leave this terminal open.

## Step 3.  Launch Rosbridge

Open a second terminal and run:

ros2 launch rosbridge_server rosbridge_websocket_launch.xml

Wait for the message "Rosbridge WebSocket server started on port 9090". Leave this terminal open.

## Step 4.  Run the Bridge Test

Open a third terminal and run:

cd ~/ros2_ws/src/course_dir

TB3_MODE=sim TB3_BRIDGE_HOST=localhost python3 bridge_test_terminal.py

The robot in Gazebo should drive forward 2m, turn right, drive 1m, turn left, drive 1m, then return. The terminal displays live LiDAR distances, odometry, and data flow status. If all three topics show OK, simulation mode is working.

## Step 5.  Run Your Algorithm

Run your algorithm the same way:

cd ~/ros2_ws/src/course_dir

TB3_MODE=sim TB3_BRIDGE_HOST=localhost python3 student_template.py

The environment variables tell the wrapper to connect to the local rosbridge and to strip the /student/ prefix from topic names so they match Gazebo’s native topics (/cmd_vel, /scan, /odom).

# 4.  Real Robot (TB3 Bridge)

Once your algorithm works in simulation, you can run the exact same code on the physical TurtleBot3. The only change is two environment variables.

## How the Bridge Works

Your code connects via WebSocket to a Raspberry Pi 5 running two ROS 2 nodes: rosbridge_websocket (translates WebSocket to ROS 2 topics) and tb3_bridge_node (safety checks, Twist to TwistStamped conversion, and topic remapping). The bridge forwards your commands to the robot and relays sensor data back to you.

## Step 1.  Set Up Phone Hotspot

Create a hotspot on your phone with:

- Name: tb3_hotspot

- Password: tb3_password

Connect your laptop to this hotspot. The TB3 and bridge will connect to it automatically.

## Step 2.  Start the TurtleBot3

Open a terminal on your laptop and SSH into the TurtleBot3:

ssh ubuntu@turtlebot3-1.local

# password: ubuntu

Then start the robot:

ros2 launch turtlebot3_bringup robot.launch.py

Wait for the LiDAR to start spinning. Leave this terminal open.

## Step 3.  Start the Bridge

Open a new terminal and SSH into the bridge Pi 5:

ssh tb3bridge@tb3bridge-desktop.local

# password: tb3bridge

if .local does not resolve, find the TB3's IP by connecting a monitor and running ip a on the device, then SSH using the IP directly.

Then start the bridge:

ros2 launch tb3_bridge tb3_bridge.launch.py

Wait for the startup banner showing the topic mapping and "Rosbridge WebSocket server started on port 9090". Leave this terminal open.

## Step 4.  Run the Bridge Test

Start your Docker container and open a terminal inside it. Then run:

cd ~/ros2_ws/src/course_dir

TB3_MODE=real TB3_BRIDGE_HOST=tb3bridge-desktop.local python3 bridge_test_terminal.py

The physical robot should drive the same pattern as in simulation. If all three data flow indicators show OK, the bridge is verified.

If the hostname doesn’t resolve, use the bridge’s IP address instead:

TB3_MODE=real TB3_BRIDGE_HOST=172.20.10.X python3 bridge_test_terminal.py

## Step 5.  Run Your Algorithm

Run the exact same code you tested in simulation:

cd ~/ros2_ws/src/course_dir

TB3_MODE=real TB3_BRIDGE_HOST=tb3bridge-desktop.local python3 student_template.py

The only difference from simulation is the two environment variables. Your Python code is identical.

## Sim vs Real Quick Reference

|  | Simulation | Real Robot |
| --- | --- | --- |
| TB3_MODE | sim | real |
| TB3_BRIDGE_HOST | localhost | tb3bridge-desktop.local |
| Needs Gazebo? | Yes (in Docker) | No |
| Needs rosbridge? | Yes (in Docker) | Runs on Pi 5 automatically |
| Needs hotspot? | No | Yes (tb3_hotspot) |
| Needs SSH? | No | Yes (TB3 + bridge) |
| Your code changes? | None | None |

## Safety (Real Robot Only)

- Publish cmd_vel at 10 Hz or faster. If you stop publishing for more than 0.5 seconds, the bridge watchdog will automatically stop the robot.

- The robot resumes when you start publishing again — no restart needed.

- Always send a zero Twist when your program exits (the template does this automatically).

- The instructor can emergency-stop the robot at any time from any machine on the network